# EnviroLab

Raspberry Pi installation and standalone hotspot setup guide

Hardware: Raspberry Pi Zero 2 W + Pimoroni Enviro+  |  Raspberry Pi OS Lite (32-bit), Debian Trixie  |  October 2026

GitHub repository: ______________________________________________

# 1. Overview

EnviroLab reads environmental sensors, displays live measurements on a Flask dashboard and the Enviro+ LCD, and records CSV data. The Pi creates its own Wi-Fi network, so classroom laptops can connect without internet access.

Final student connection: Wi-Fi SSID EnviroLab  •  Dashboard http://10.42.0.1:5000  •  SSH pi@10.42.0.1

# 2. Hardware and prerequisites

Raspberry Pi Zero 2 W, microSD card, suitable USB power supply, Pimoroni Enviro+ HAT and 40-pin connection/ribbon cable.

No PMS5003 particulate sensor is required. Gas readings are raw resistance values; sound is a relative measurement, not calibrated dB.

For initial setup, connect the Pi to an existing Wi-Fi network and use SSH. Keep a local recovery method available before enabling the hotspot.

# 3. Install Raspberry Pi OS and connect

Flash Raspberry Pi OS Lite (32-bit) to the microSD card using Raspberry Pi Imager. Configure hostname envirolab, username pi, SSH and your initial Wi-Fi credentials in the Imager.

Boot the Pi, then connect from Windows PowerShell:

ssh pi@envirolab.local

Update packages and reboot if necessary:

sudo apt update
sudo apt full-upgrade -y
sudo reboot

The example paths below assume user pi and project folder /home/pi/envirolab. Adjust if your installation differs.

# 4. Enable the hardware interfaces

Enable I²C and SPI using raspi-config if they are not already enabled:

sudo raspi-config

Under Interface Options, enable I2C and SPI. The Enviro+ LCD also needs the SPI pin configuration below.

# 5. Install the Pimoroni software

Install Git and the libraries used by the Enviro+ example:

sudo apt install -y git python3-venv libopenblas0-pthread libopenjp2-7 libfreetype6 fonts-dejavu-core

Clone Pimoroni’s Enviro+ Python repository and run its installer:

cd ~
git clone https://github.com/pimoroni/enviroplus-python.git
cd ~/enviroplus-python
./install.sh

The working installation uses the virtual environment at ~/.virtualenvs/pimoroni. Confirm it exists before proceeding:

ls ~/.virtualenvs/pimoroni/bin/python

# 6. Configure SPI for the Enviro+ LCD

Edit the Pi boot configuration:

sudo nano /boot/firmware/config.txt

Comment out the generic SPI enable line, if present, and use this overlay instead:

#dtparam=spi=on
dtoverlay=spi0-2cs,no_miso

This preserves SPI chip-select 1 for the LCD and frees GPIO9 for its D/C signal. Reboot after editing:

sudo reboot

Check that /dev/spidev0.1 exists and that the Pimoroni example displays correctly:

ls /dev/spidev*
~/.virtualenvs/pimoroni/bin/python ~/enviroplus-python/examples/all-in-one.py

# 7. Install EnviroLab from GitHub

Once the code is published, replace the placeholder below with the repository URL:

cd ~
git clone <GITHUB_REPOSITORY_URL> envirolab
cd ~/envirolab

Install project requirements into the existing Pimoroni virtual environment, if the repository supplies a requirements.txt:

~/.virtualenvs/pimoroni/bin/python -m pip install -r requirements.txt

The project contains main.py, config.py, sensors/, logging_data/, web/, display/ and data/. Chart.js is served locally from web/static/chart.umd.js; do not replace it with an internet-only CDN link.

# 8. Check EnviroLab configuration

Open config.py and confirm the relevant settings (names shown as used in the working project):

MOCK_SENSORS = False
SAMPLE_INTERVAL = 5.0
WEB_HOST = "0.0.0.0"
WEB_PORT = 5000
LCD_ENABLED = True
LCD_ROTATION = 270  # if using the configurable rotation setting

The SENSORS dictionary determines which measurements are enabled. Enable light, proximity and sound only as required. The sound value is relative, not dB; gas values are uncalibrated resistance measurements.

The LCD uses the selected dashboard measurement. The working ST7735 setup uses SPI CS1, GPIO9 (D/C), GPIO12 (backlight), and rotation 270°.

# 9. Test the application manually

cd ~/envirolab
~/.virtualenvs/pimoroni/bin/python main.py

From a laptop on the same initial Wi-Fi network, open http://envirolab.local:5000 (or the Pi’s LAN IP). Check live readings, graph selection, LCD updates, and start/stop/download CSV. Stop with Ctrl+C.

# 10. Configure automatic startup with systemd

Create /etc/systemd/system/envirolab.service:

sudo nano /etc/systemd/system/envirolab.service

[Unit]
Description=EnviroLab Environmental Monitoring
After=network.target

[Service]
Type=simple
User=pi
WorkingDirectory=/home/pi/envirolab
ExecStart=/home/pi/.virtualenvs/pimoroni/bin/python /home/pi/envirolab/main.py
Restart=on-failure
RestartSec=5
Environment=PYTHONUNBUFFERED=1

[Install]
WantedBy=multi-user.target

Enable, start and inspect the service:

sudo systemctl daemon-reload
sudo systemctl enable --now envirolab.service
sudo systemctl status envirolab.service

For live application logs:

journalctl -u envirolab.service -f

# 11. Configure the standalone Wi-Fi hotspot

NetworkManager is used on Raspberry Pi OS Trixie. Check the Wi-Fi device:

nmcli device status

Create a 2.4 GHz access point named EnviroLab:

sudo nmcli connection add type wifi ifname wlan0 con-name EnviroLab ssid EnviroLab
sudo nmcli connection modify EnviroLab \
  802-11-wireless.mode ap \
  802-11-wireless.band bg \
  ipv4.method shared \
  ipv4.addresses 10.42.0.1/24 \
  ipv6.method disabled

Set a unique WPA2 password (minimum eight characters). Do not commit the password to GitHub:

sudo nmcli connection modify EnviroLab \
  wifi-sec.key-mgmt wpa-psk \
  wifi-sec.psk "REPLACE_WITH_A_STRONG_PASSWORD"

Make the hotspot the preferred connection at boot:

sudo nmcli connection modify EnviroLab \
  connection.autoconnect yes \
  connection.autoconnect-priority 100

WARNING: The next command will disconnect SSH if you are connected over the Pi’s original Wi-Fi. Have a recovery route before proceeding.

sudo nmcli connection up EnviroLab

Connect a laptop to the EnviroLab SSID, then visit http://10.42.0.1:5000. Windows may display “No internet, secured”; this is expected. To SSH while on the hotspot:

ssh pi@10.42.0.1

# 12. Final power-cycle test

Reboot the Pi, or shut down and power it from an ordinary USB power supply.

Wait for the EnviroLab Wi-Fi network to appear, then connect with the password.

Open http://10.42.0.1:5000 and confirm live measurements.

Change the dashboard measurement and confirm the LCD follows it.

Start recording, wait for several readings, stop and download the CSV.

# 13. Troubleshooting

Service not starting: sudo systemctl status envirolab.service; journalctl -u envirolab.service -n 50 --no-pager

Hotspot not visible: nmcli device status; nmcli connection show --active; sudo nmcli connection up EnviroLab

LCD blank or incorrect: verify /boot/firmware/config.txt overlay, /dev/spidev0.1, GPIO9/12, CS1 and rotation 270.

BME280 test: use smbus2.SMBus(1) and bme280.BME280(i2c_dev=bus); the enviroplus.weather import is not valid for this installation.

Light/proximity reads zero: use an LTR559 instance (from ltr559 import LTR559), with a brief 0.5 s settling delay after construction.

Sound module: get_noise_profile is a method on an instance of enviroplus.noise.Noise, not a module-level function. Return an ordinary Python float for JSON/CSV compatibility.

Dashboard shows no readings: ensure Flask callbacks are wired from main.py to web/server.py; do not import main from the server module, which can create a second copy of the application state.

# 14. Maintenance

Update application code after publishing to GitHub:

cd ~/envirolab
git pull
sudo systemctl restart envirolab.service

To stop or disable automatic launch:

sudo systemctl stop envirolab.service
sudo systemctl disable envirolab.service

CSV recordings are saved under the project’s data/ directory. Back up any required recordings before re-imaging the microSD card.

Security: the hotspot is intended for local classroom use. Use a strong Wi-Fi password, keep SSH credentials private, and do not expose Flask directly to the public internet.